Benha University

Benha Higher Institute of Technology

# Department of Mechanical Eng.

Subject : Turbo machines     Date: 17/1/2011

###### Model Answer of The Final Exam

Elaborated by: Dr. Mohamed Elsharnoby

المادة : ألات  تربينية

1-a) Fan is a machine imparting only a small pressure rise to continuously flowing gas; usually the density ratio across the fan is less than 1.05 such that the gas is considered to be incompressible. While the density ratio across the compressor exceeds 1.05 and the flow should be considered as compressible one.

ii) Impeller is the rotating member in the centrifugal pump or compressor; but the runner is the rotating part in radial flow hydraulic turbine or pump.

iii) Both diffuser and draught tube are passages that increases in cross-sectional area in direction of fluid flow so that they converts the kinetic energy into static pressure head. While the diffuser is usually situated at the outlet of a compressor; the draught tube is usually situated at the outlet of a hydraulic turbine.

b)   The power dimensionless specific speed, Nsp, is found by eliminating the diameter D from the power and head coefficients.

The power and flow coefficients are respectively given by:P=P/ρN3D5 , ψ=gH/N2D2so we get Nsp=P1/2/ψ5/4=NP1/2/ρ1/2(gH)5/4.Figure 1 shows a typical hydraulic turbine runner shapes for different specific speeds along with their optimum or design efficiencies. Its observed that Pelton is used for low specific speed , Francis turbine for moderate specific speeds and axial flow turbine for high specific speeds.

Fig.1 Variation of hydraulic turbine runner design with dimensionless specific speed.

1-c) Assuming dynamic similarity exists between the firdt and second sized pumps, we equate the flow coefficients. Thus

From Fig.2 at Q1 = 2.83 L/s and 2000 rpm the head H1 is 14 m and equating the head coefficients for both cases gives,

Fig.2

2-a) A typical pumping system is shown in Figure 3 where

Figure 3. Pumping system

b) the dependence of pump characteristics on blade outlet angle β2are shown on Figures 4 and 5.

Figure 4 shows the theoretical dependence between the energy head and the flow rate Q which is given by the equation E=U22/g−(QU2cotβ2/gA)

Figure 4 : Theoretical characteristics for varying blade outlet angle

Figure 5 Actual characteristics for varying blade outlet angle

Figure 5 shows the actual dependence of power and head on the flow rate for different β2.

c) The single pump     characteristic is plotted in the figure below along with the characteristics for the two pumps connected in series and in parallel. Since the same pump is used in both cases ; for the series connection the flow rate is the same and the head is doubled while for the parallel connection the head  across the pump is the same and the flow rate is doubled.

Figure 6: Axial pump characteristics when connected in parallel and series.

Series connection

Parallel connection

At point A both connections give the same head and flow rate and the system characteristics must pass through this point and the point C(0.2) since there is 2m static lift.

At operating point A in the above figure

Q = 0.48 m3/sec,    H = 9.75

For the system curve          H = Hs + K Q2

But Hs = 2 , therefore         H = 2 + 0.482 K = 9.75

And    then                          K = 33.64

Hence the system curve is given by :

H = 2  + 33.64 Q2

The system curve is parabolic and may be drawn for various flow rates and head as shown in the above figure. The point B is the operating point for a single pump within the system.

The single pump operates at point B :

Q = 0.4 m3/sec

H = 7.4 m

3-a) Figure 7 illustrates the system with the velocity triangles.

Figure 7 Inlet velocity triangle

The hydraulic efficiency is given by:

3-b)The hydraulic efficiency is given by

P = 4578 kW (this power is delivered to runner)

4-a) b) Figure 8  illustrates the change in total pressure loss coefficient and mean deflection on changing the angle of incidence i

Figure 8 : cascade mean deflection and pressure loss curves.

And substituting for (h2-h1), (h3-h1), the reaction ratio R becomes

b) The inlet Mach number should be restricted to an acceptable value . It may be achieved by placing guide vanes at the inlet . Guide vanes at the inlet will impart a whirl component Cx1 to the fluid, thus reducing W1 to an acceptable value. This is shown in the figure  9 below.

Figure 9: The effect of inlet guide vanes on the inlet relative velocity: (a) at shroud; (b) at hub

4-c) The diagram temperature difference is

\

= 167.2 degree

Therefore U2 = 425.75 m/sec

D2 = 0.6776  m

The impeller diameter is equal to 0.6776 m

The overall loss is proportional to (1-ηc) =(1-0.8). Half of the overall loss is therefore 0.5(1-0.8) = 0.1 and therefore the effective efficiency of the impeller in compression is (1-0.1) = 0.9.

= 4.35

To2 = 288 + 167.2 =455.2 K

T2 = 379.3 K

P2/Po2 = 0.5283

P2 = 232.8  kPa

ρ2 =2.138 kg/m3

To find the flow velocity normal to the prephery of the impeller

Cx2=σ U2

= 0.9 x 425.75 = 383.2 m/sec

The Mach number is one at exit so

C22=γRT2

C22=1.4×287.1×379.2=152415.7

C22=Cx22+Cr22

Cr2=74.66m/sec

A2=mρ2Cr2=82.138×74.66=0.05012

The impeller depth b at outlet is given by:

b=A2πD2=0.023545m

5-a)If we compress in a single compression from 1 to 5, the isentropic work done is

Figure 10

5-b)In a rotor of a turbine or compressor the total relative enthalpy is constant from inlet to outlet i.e.

h01rel=h02rel.

Then       h1+W122=h2+W222

Or  h1−h2=W22−W122

Across the stage the fluid may be considered as incompressible. Therefore

p01rel=p+ρW122  and  p02rel=p+ρW222

Hence  h1−h2=p02rel−pρ−p01rel−pρ

Now along an isentropic process

Tds=dh−1ρdp=0

Therefore dh=1ρdp

Along 1-2s    h2s−h1=p2−p1ρ

Thus h2−h2s=p01rel−p−p02rel+pρ−p2−p1ρ

h2−h2s=p01rel−p02relρ

5-c) The reaction ratio R is given by:

R=φ(tanβ1+tanβ2)/2

Where,

φ=CaU

U=ωr=2πn60r

=2π×18000600.18=339.292m/sec

∴φ=0.530516

(tanβ1+tanβ2)=2Rφ=1.8849555

∵β2=25o→→∴tanβ1=1.4186

∴β1=54.82o→→(i)

α2=β1=54.82o

m=ρCaπ4(D02−Di2)

ρ=pRT=1.18877kg/sec

m=1.18877×180×π4(0.422−0.32)

=14.52kg/sec→→(ii)

Actual Work =λUCa(tanβ1−tanβ2)=51182J/kg

Power required =51182ηηm=904.146kW→→(iii)

ψ=λ(tanα2−tanα1)

ψ=0.4446

T01=T1+C122CP=312.62K

T03=363.55K=T02

P02Po1=(1+0.88(363.55−312.62)312.62)γγ−1

P02Po1=1.598→→(iv)

M1=W1a=Cacosβ1γRT

M1=0.91→→(v)